12 July 1997

This update of the P112 ROM incorporates all known updates provided by Dave

Brooks as of mid-June 1997.  It also adds the following features:

- Configuration of Non-Volatile Clock RAM settings with default

values selected by simply entering a Carriage Return.

- Setting and Displaying of the Date/Time from the DS-1202 Clock.

The Year will be accurate over the range 1978-2076 by assuming that

year values 78..99 are in the twentieth century (19xx), and years

in 00..77 are in the twenty-first century (20xx).  Setting of the

date may be entered as; Month/Day/Year -or- Day.Month.Year.  The key

to interpreting the values is the separator ("/" or ".").

- A slight delay is included prior to the Boot ROM code setting wait

state delays as a safety valve in case an inadequate number of wait

states is configured.  The delay allows one to enter the Setup

routine directly to avoid locking the system (selected by pressing

ESCape at the pause prompt).  The delay also allows direct entry into

the Debugger by pressing the Enter (Return) key.  If no key is

pressed, the boot sequence will start after approximately a 6 second\

pause.

- To accomodate the new commands, the Boot command from the Debugger

was changed from "S" (which is now the SETUP command) to "Z" which

can stand for whatever you desire.  Arguments allowed with this

command are:

T	Attempt to boot from the first available device

(Floppy 0, SCSI 0, GIDE (IDE) Master).

Z 1	Attempt to Boot Floppy Unit 0 only.

Z 2	Attempt to Boot SCSI Unit 0 only.

Z 3	Attempt to Boot GIDE (IDE) Master only.

The "T" command displays the Time and Date, while "T S" prompts

you for information with which to set the Time and Date.

The files comprising this version of the bios have been successfully assembled

with the SLR180+ (no longer available) and Al Hawley's ZMAC assembler.

Harold "Hal" Bower

12 July 1997

####################################################